These are industrial desalination systems, typically used for cities, energy plants, or research bases:

1. Intake and Pre-Treatment Systems

Before desalination, seawater must be filtered and conditioned to remove debris, algae, and sediments.

| Equipment | Function |
| --- | --- |
| Seawater Intake Pumps | Draw seawater into the plant from ocean or dam-side intake. |
| Coarse Screens & Filters | Remove large solids like shells, sand, and debris. |
| Microfiltration / Ultrafiltration Units | Fine filtering to protect high-pressure pumps and membranes. |
| Chemical Dosing Systems | Add anti-scalant and chlorine for biological control. |

2. Desalination Core Systems

Two main technologies are used here:

A. Reverse Osmosis (RO) System – Most Common

| Equipment | Function |
| --- | --- |
| High-Pressure Pumps (1000–1500 psi) | Force seawater through semi-permeable RO membranes. |
| RO Membrane Modules | Main purification unit—removes 99% of salt and impurities. |
| Energy Recovery Devices (ERDs) | Reuse pressure energy from brine to save power. |
| Pressure Vessels | Hold the membrane elements under high pressure. |

B. Thermal Desalination (used near power or dam sites)

| Process | Equipment |
| --- | --- |
| MSF (Multi-Stage Flash Distillation) | Flash chambers, brine heaters, condensers. |
| MED (Multiple Effect Distillation) | Series of evaporators and condensers working at reduced pressures. |
| Vapor Compression Units | Compress and recycle steam for efficient heat use. |

3. Post-Treatment & Storage

After desalination, the water must be stabilized and stored.

| Equipment | Function |
| --- | --- |
| Remineralization Units | Add minerals (like calcium, magnesium) back to make water healthy. |
| pH Adjustment Systems | Control acidity using CO₂ or lime. |
| Disinfection Units (UV or Chlorine) | Ensure no bacteria remain. |
| Fresh Water Storage Tanks / Reservoirs | Store processed water for supply. |

4. Brine Management Systems

Salt concentrate (brine) needs safe handling.

| Equipment | Function |
| --- | --- |
| Brine Disposal Pumps | Send concentrated brine to safe outfall zones or evaporation ponds. |
| Evaporation Ponds / Crystallizers | Recover salt and minerals. |
| Diffuser Systems | Disperse brine safely back into seawater. |

5. Power & Monitoring

| Equipment | Function |
| --- | --- |
| Electric Control Panels (PLC/SCADA) | Automated plant operation and monitoring. |
| High-Capacity Generators / Solar Systems | Power source for pumps and control systems. |
| Instrumentation Sensors | Pressure, salinity, pH, turbidity, temperature, and flow sensors. |

6. Supporting Infrastructure

| Equipment | Function |
| --- | --- |
| Concrete / Steel Tanks | For intake and treated water. |
| Pipelines & Valves | Distribute and regulate flow. |
| Maintenance Tools | Membrane cleaning systems (CIP units), chemical mixers, etc. |

Example of a Typical Setup Near a Dam

If  building a hybrid desalination station beside a dam, than may combine:

- Seawater intake from nearby coastal area,

- Reverse Osmosis desalination module,

- Freshwater outlet connected to dam’s storage basin,

- Solar or turbine-powered pumping system for sustainable energy.